ns2 - Network Simulator Version 2

Abu Asaduzzaman & Dr. Imad Mahgoub

{ aasaduzz@cse.fau.edu & imad@cse.fau.edu }

Department of Computer Science and Engineering

Florida Atlantic University

Mobile Computing, Fall 2003

Abstract

Network Simulator (NS) is a simulation tool targeted at both wired and wireless (local and satellite) networking research. NS is a very promising tool and is being used by universities and researchers. In this report we provided information how to install NS2 on Unix and Windows. Then we discussed how to use NS2 to simulate wired and wireless networks. A simple but limited method is to combine the existing components with OTcl scripts; a complex but powerful method is to implement new components into NS2 using C++. We discussed both of the two methods. Finally, the methods to animate (using NAM) and to analyze (using Xgraph/GNUplot) the simulation results are presented.

Keywords: Network simulator, NS2, wired network, wireless network, routing protocol, NAM, Xgraph.

# Introduction

Network Simulator (NS), a discrete event simulator targeted at networking research, is widely used in universities and companies by researchers. NS provides substantial support for simulation of TCP, routing, and multicast protocols over wired and wireless (local and satellite) networks. The latest version of NS is 2.26 (NS2). It implements network protocols such as FTP and Telnet, routing algorithms such as SPF and DV, and 'lower' layers such as logic link (LL) and media access control (MAC). NS began as a variant of the REAL network simulator in 1989. In 1995 NS development was supported by DARPA through the VINT project at LBL, Xerox PARC, UCB, and USC/ISI. Currently NS development is support through DARPA with SAMAN and through NSF with CONSER. NS has always included substantial contributions from other researchers, including ACIRI, UCB, CMU, and Sun Microsystems. [1]

Nam began at LBL. The Nam development effort was an ongoing collaboration with the VINT project. Currently, it is being developed at ISI as part of the SAMAN and Conser projects. [2]

The simple way NS2 can be used is for studying the property of a well-known protocol. In this case, a script language OTcl is used to glue the network components (nodes, links, agents, applications, etc) provided by NS2, configure the parameters (band-width, delay, routing protocol, etc) and launch activities (data transfer, topology change, etc). NS2 will read these configurations; simulate each network event, and record events and statistics in to trace files. After the simulation, Nam can demonstrate the events in a visualized way. For the simple usage of NS2, an understanding to the simulation framework is necessary.

When NS2 is used to simulate a new protocol, e.g., an ad hoc wireless routing protocol, the simple way is not enough. The advanced way is to hack the source of NS2 with C++, define new network components, rebuild the whole system and run the customized version of NS2. For the advanced usage of NS2, knowledge of the simulator implementation is required. [3]

NS2 provides a collaborative environment. It is freely distributed. Expert developers came up with the shared codes, protocols, models, etc. Our purpose to study NS2 as a part of the class project is to learn how to

(1) Introduction

- Install NS2 in Unix and Windows

- Use it to simulate wired and wireless networks

- Implement new components into NS2 using C++

(5a) Use NAM to visualize the simulation results

(5b) Use Xgraph to analyze the simulation results

- Use some special techniques for wireless network simulation

- Conclusion

Appendix A

This report is organized as follows: Section 1 has the introduction about NS2 and our goals to study NS2. Section 2 provides information regarding NS2 installation in Unix and Windows. Section 3 introduces the principles and concepts of network simulation via NS2. Section 4 discusses the structures of NS2 that are essential for implementation of new routing protocols. Section 5 shows the results produced in the simulation and how to use them with Nam and Xgraph/GNUplot. Section 6 covers some special aspects for wireless network simulation. Section 7 concludes this report. The appendix A, at the end, includes some of the Internet references.

2	NS2 Installation

2.l	UNIX/Linux

Install NS2 under UNIX environment is very straightforward. The only requirement is large disk space and rapid Internet access. The all-in-one source packages for NS2 are 33M, and after unpack and compilation, the final disk occupation will reach 100M.

Here are the steps to build NS2 in UNIX/Linux:

- Make sure UNIX/Linux is installed to support NS2 (has enough

memory space, C++ compiler, etc)

2.	Download the all-in-one source package from the NS2 download

page: http://wvw.isi.edu/nsnam/ns/ns-build.html#allinone

3.	Unpack the source package:

gunzip -c ns-allinone-2.26.tar.gz I tar -xvf -

- Install: cd ns-allinone-2.26; ./install

- Set PATH environment variable

- Validate: cd ns-2.26; ./validate

The install program will run quite a well. When finished, all is done. The binary files including ns (for NS2) and nam (for NAM) will be created in corresponding directory, and you can run a sample simulation via: ns tcl/ex/simple.tcl.

2.2	Windows

Installing NS2 under Windows (9x/ME/NT/2000/XP) requires additional software such as Cygwin and XFree86 and more experience. Cygwin and XFree86 can be downloaded (free of cost) from Internet. [4][5] The all-in-one NS2 source package can be downloaded from Internet and unpacked. [6]

Here are the steps to build NS2 in Windows:

- Make sure system has enough memory spaces

- Install Cygwin-1.3.12 or higher with UNIX text type

- Install XFree86 binaries, sources, and configuration files

- Make sure make, patch, tar, gzip, etc works fine

5.	Download the all-in-one source package from the NS2 download

page: http://wvw.isi.edu/nsnam/ns/ns-build.html#allinone

6.	Unpack the source package:

gzip -d -c ns-allinone-2.x.tar.gz | tar xvf –

mv nam-1.9.configure ns-allinone-2.26/nam-1.9/configure

- Install: cd ns-allinone-2.26; ./install

- Set PATH environment variable

- Validate: cd ns-2.26; ./validate

The install program will run quite a well. When finished, all is done. The binary files including ns (for NS2) and nam (for NAM) will be created in corresponding directory, and you can run a sample simulation via: ns tcl/ex/simple.tcl.

3	Simulation Framework [8]

NS2 simulation is about the processing of packets. Packets are transported among network nodes via network links. These packets are generated by network applications (data packet) or network protocols (control packet). An event scheduler is in charge of ordering all the packets (or events) by their arriving time and let nodes, links and agents to handle these packets in sequence. A full trace of each packet or their statistics information can be stored in a trace file, and used to generate graphs or animations.

In this section, the concept of node, link, agent, and scheduler will be introduced, as well as their usage in an OTcl script to configure a specific NS2 simulation. The usage of trace files and network animator (Nam) will be discussed in Section 4.

3.1	Nodes and Links: Network Topology

In a simplified model, network topology in NS2 consists of nodes, links, agents, applications, etc as shown in Figure 1. Links connect nodes in pairs. For LAN or wireless simulation, there are also channels that can be shared by more than two nodes. We will discuss them later.

CBR

- Nodes

- Hosts, Routers

2

1

- Links

- Queues

- Agents

- Protocol

- Applications

- CBR

3

4

5

UDP

UDP Sink

# Figure 1: NS Communication Model

The following script segment creates two nodes. When a node is created, it is automatically assigned an address and a default routing module. Nodes are the sources and destinations of the packets. In the script, $ns is the scheduler object and node is its method to create a node, $n0 and $nl hold the reference to the newly created node object.

set n0 [$ns node]

set ni [$ns node]

The following script segment creates one link to connect these two nodes with lM bandwidth and l0ms delay. A link receives packets from one end, compute the delay and transmission time, and send them to the other end after the time elapsed. If two or more packets are going through the same link, the later packet should wait in a queue until all previous packets are sent. If the number of packets waiting exceeds the queue limit, new packets sent to this link are dropped.

$ns duplex-link $n0 $n1 1Mb 10ms DropTail

The default routing algorithm is centralized link state algorithm (like Dijkstra's SPF). The route from one node to each other node is computed by the simulator and stored in the routing table of this node. From user's point of view, each node just 'know' how to reach another node in the network.

3.2	Agents: Network Protocol and Application

When the network topology is created, agents can be attached to nodes to generate packets and put them onto the network. There are agents in different network protocol layers. In the transportation layer, there are TCP and UDP agents. The following OTcl script segment create a UDP agent and put it on node $n0. When an agent is attached to a node, it send and receive packet through this node.

set udp0 [new Agent/UDP]

$ns attach-agent $n0 $udp0

In the application layer, there are agents for well known protocols like FTP, Telnet, HTTP, etc; there are also special agents for simulation use, like CBR, which generate continues data streams. The following OTcl script-segment creates a CBR agent and attaches it to the UDP agent. When an agent is attached to another agent, it sends and receives data through this agent.

set cbr0 [new Application/Traffic/CBR]

$cbr0 attach-agent $udp0

Agents are usually created in pairs, put in different nodes and exchange data between each other. The following OTcl script segment creates a Null agent on node $n1 and connect it to the UDP agent on node $n0. After the agents are connected, the data generated by the CBR agent and passed to the UDP agent will go though node $n0, the link between node $n0 and $n1, node $n1, and received by the Null agent. The Null agent is like a null device, it discards any packet it receives.

set null0 [new Agent/Null]

$ns attach-agent $n1 $null0

$ns connect $udp0 $null0

There are also agents in other layers, e.g., the router agents when dynamic routing is used. However, these agents are always hidden in the inner structure of nodes, and created automatically by the simulator.

3.3	Scheduler: Packet Is Event

If nodes and links is the scene in the stage, agents are the actors; then the scheduler is the director. The scene and actors don't know how to perform unless the director tells them. The scheduler is a build-in object in NS2 user can tell it when to do what. The following OTcl script segment asks the scheduler to open the traffic generator at 0.5 second of the simulation and close it at 4.5 second. The script segment also told the scheduler the whole simulation will last for 5.0 seconds.

$ns at 0.5 "$cbr0 start" $ns at 4.5 "$cbr0 stop"

$ns at 5.0 "finish"

Actually, a scheduler's work is much more than that. Every time a packet is received by an agent or a link, a delay time representing the processing time or network delay is computed and actually the packet is handed to the scheduler as an event. The scheduler orders all the events by time and fires them one by one; i.e., when the time comes, the agent or link will send the early received packet to its destination. There are also other events than packets, e.g., a TOP agent may set a timer for its time window and re-send a packet when timeout.

$ns run

The above OTcl script segment will start the simulation; i.e., the scheduler begins to fire the events one by one, according to their time stamps. In NS2, the scheduler is non-real-time by default. The time stamps are not physical time in the simulation, but used to order the events. So a 5.0-second simulation may take 2.0 seconds or 1 hour, de- pending on the complexity of the topology and the number of events fired during the simulation.

3.4	The Complete Sample Script

The following OTcl script 'sample.tcl' will create the network topology and agents we discussed before, start the simulation, and display an animation of the simulation. The beginning part of the script creates the simulator object, opens a trace file, and tells the simulator to launch Nam when the simulation is over. You can run the script by typing: ns sample.tcl.

#Create a simulator object

set ns [new Simulator]

#Open the nam trace file set nf [open out.nam w]

$ns namtrace-all $nf

#Define a 'finish' procedure

proc finish {} {

global ns nf

$ns flush-trace #Close the trace file

# close $nf

#Execute nam on the trace file

exec nam out.nam &

exit 0;

}

# Create two nodes

set n0 [$ns node]

set nl [$ns node]

# Create a duplex link between the nodes

$ns duplex-link $n0 $n1 1Mb 10ms DropTail

# Create a UDP agent and attach it to node n0 set udp0 [new Agent/UDP]

$ns attach-agent $n0 $udp0

# Create a CBR traffic source and attach it to udp0

set cbr0 [new Application/Traffic/CBR]

$cbr0 set packetSize_ 500

$cbr0 set interval_ 0.005

$cbr0 attach-agent $udp0

# Create a Null agent (a traffic sink) and attach it to node n1

set null0 [new Agent/Null]

$ns attach-agent $n1 $null0

#Connect the traffic source with the traffic sink

$ns connect $udp0 $null0

#Schedule events for the CBR agent

$ns at 0.5 "$cbr0 start"

$ns at 4.5 "$cbr0 stop"

#Call the finish procedure after 5 seconds of simulation time

$ns at 5.0 "finish"

#Run the simulation

$ns run

3.5	Screen-shot from NAM

Screen-shot taken after simulation (not real) time 0.6 second, Figure 2, packets are being transferred from node n0 to node n1.

# Figure 2: Screen-shot from NAM

4	Simulator Extensions [8]

From technical point of view, NS2 is basically an OTcl interpreter with network simulation object libraries. If a needed network object is not there, users can write the object with C++ and add it into the library.

4.1	Tcl/OTcl

Tcl stands for the Tool Command Language. Otcl stands for MIT Object Tcl. Tcl has the ability to be embedded into an application system, say, an editor or a simulator, and act as an interface language which allow user to invoke application specific functions with Tcl commands. In other words, Tcl is extendable; you can add all your desired functions into its library.

Tcl is a interpreted script language with very simple syntax. For example:

set a 43

will store number 43 into variable $a, and

set b [expr $a * 2 + 1]

will compute the expression $a * 2 + 1 and store the result into $b.

OTcl is an extension to Tcl for object- oriented programming. In OTcl, users can define classes and create ob- jects. For example:

Class guest

guest instproc greet {} {

$self instvar name_

puts "Hello, $name_!"

}

will create a new class guest and install a method greet for it to say hello to the guest, using the guest name stored in a member variable $name-. And the following OTcl script will create a instance of guest, set its member variable, and invoke its method:

set a [new guest]

$a set name- "Tim"

$a greet

OTcl class can also be defined in C++. Actually sometime it is necessary to implement the critical network components in C++ for the sake of efficiency. In the following subsections, an example of implementing a simplified ping protocol is used to demonstrate how to add new agents to NS2 using C++.

4.2 Add New Packet Header

The new ping protocol needs a new type of packet. In the implementation of NS2, actually all packet have the same format, containing the packet headers for EVERY protocol. Different protocols read and write different packet headers and ignore the packet headers for other protocols. So adding a new packet type is adding a new packet header into the common packet structure. The following program segment is the definition of the ping packet head:

struct hdr_ping {

char ret;

double send_time;

};

After defining the packet header format, the following program segment tells NS2 that this new packet header should be added into the common packet, and should be referred in OTcl with the name Packet- Header/Ping.

static class PingHeaderClass : public PacketHeaderClass {

public:

PingHeaderClass() : PacketHeaderClass("PacketHeader/Ping",

sizeof(hdr_ping» {}

} class-pinghdr;

The next step is to change NS2's packet type list to add the new packet type:

enum packet_t {

PT_TCP,

PT-UDP,

… …

// insert ney packet types here

PT-PING, // packet protocol ID for our ping-agent

PT-NTYPE // This MUST be the LAST one

};

and give it a name in the packet information list. This name will be displayed in the packet trace file.

class p-info { public:

p-info() {

name-[PT-TCPJ= "tcp";

name-[PT-UDPJ= "udp";

name-[PT-PING]="Ping";

name-[PT-NTYPE]= "undefined";

}

… …

}

Now, a new packet type PT.PING is available to the new ping protocol.

4.3	Add New Agent

The new agent is defined as a subclass of NS2 class Agent, and all functions fulfilled by the PingAgent are defined in methods command and recv. The command method accepts and carries out commands from OTcl script, e.g., sends a ping packet; the recv method handles the packets sent to the agent, e.g., replies a ping packet.

class PingAgent : public Agent {

public:

PingAgent();

int command(int argc, const char*, const* argv);

void recv(Packet*. Handler*);

protected:

int off_ping_;

};

The next thing to do is to bind the C++ class PingAgent to the OTcl class Agent/Ping:

static class Ping Class : public TclClass {

public:

Ping Class() : TclClass("Agent/Ping") {}

TclObject* create(int, const char*const*) {

return (nev PingAgent());

}

} class_ping;

And the member variables of the C++ class are bound to the OTcl member variable, so that they are also accessible in the OTcl script.

PingAgent: :PingAgent() : Agent(PT-PING) {

bind("packetSize_", &size_);

bind("off_ping_", &off_ping_);

}

The source codes for the command and recv methods are listed in the next subsection. After the new C++ code are compiled and combined into the original NS2 code, a new agent Agent/Ping can be create in OTcl script, put on two nodes, and ping each other to test the network round- trip delay.

3.4	The Complete Sample Program

/*

* File: Header File for a new 'Ping' Agent Class for the ns

* network simulator

* Author: Marc Greis (greis@cs.uni-bonn.de), May 1998

*/

#include "agent.h"

#include "tclcl.h"

#include "packet.h"

#include "address.h"

#include "ip.h"

struct hdr_ping {

char ret;

double send_time;

};

class PingAgent : public Agent {

public:

PingAgent(};

int command(int argc, const char*const* argv};

void recv(Packet*. Handler*};

protected:

int off_ping_;

};

/*

* File: Code for a new 'Ping' Agent Class for the ns * network simulator

* Author: Marc Greis (greis@cs.uni-bonn.de). May 1998

*/

#include "ping.h"

static class PingHeaderClass : public PacketHeaderClass {

public:

PingHeaderClass() : PacketHeaderClass("PacketHeader/Ping",

sizeof(hdr_ping)) {}

} class_pinghdr;

static class PingClass : public TclClass {

public:

Ping Class() : TclClass("Agent/Ping") {}

TclObject* create(int, const char*const*) {

return (nev PingAgent());

}

} class_ping;

PingAgent: :PingAgent() : Agent(PT_PING) {

bind("packetSize_", &size_);

bind("off_ping_", &off_ping_);

}

int PingAgent: :command(int argc. const char*const* argv) {

if (argc == 2) {

if (strcmp(argv[l] , "send") == 0) {

// Create a nev packet

Packet* pkt = allocpkt();

// Access the Ping header for the nev packet:

hdr_ping* hdr = (hdr_ping*)pkt->access(off_ping_);

// Set the 'ret' field to 0, so the receiving node knows

// that it has to generate an echo packet

hdr->ret = 0;

// Store the current time in the 'send_time' field

hdr_>send_time = Scheduler: :instance() .clock();

// Send the packet

send(pkt, 0);

// return TCL-OK, so the calling function knows that the

// command has been processed

return (TCL-OK);

}

}

// If the command hasn't been processed by PingAgent(}: :command,

// call the command() function for the base class

return (Agent: :command(argc, argv}};

}

void PingAgent: :recv(Packet* pkt, Handler*) {

// Access the IP header for the received packet:

hdr_ip* hdrip = (hdr_ip*)pkt->access(off_ip_);

// Access the Ping header for the received packet:

hdr_ping* hdr = (hdr_ping*)pkt->access(off_ping_);

// Is the 'ret' field = 0 (i.e. the receiving node is being pinged)?

if (hdr->ret == 0) {

// Send an 'echo' .First save the old packet's send_time

double stime = hdr->send_time;

// Discard the packet

Packet: :free(pkt);

// Create a new packet

Packet* pktret = allocpkt();

// Access the Ping header for the new packet:

hdr_ping* hdrret = (hdr_ping*)pktret->access(off_ping_);

// Set the 'ret' field to 1, so the receiver won't send another echo

// hdrret->ret = 1;

// Set the send_time field to the correct value

hdrret->send_time = stime;

// Send the packet

send(pktret, 0);

} else {

// A packet was received. Use tcl.eval to call the Tcl

// interpreter with the ping results.

// Note: In the Tcl code, a procedure 'AgentlPing recv {from rtt}'

// has to be defined which allows the user to react to the ping

// result.

char out[100];

// Prepare the output to the Tcl interpreter. Calculate the round

// trip time

sprintf(out, "%s recv %d %3.1f", name(),

hdrip->src_ >> Address: :instance().NodeShift_[1],

(Scheduler: :instance().clock()-hdr->send_time) * 1000);

Tcl& tcl = Tcl: :instance();

tcl.eval(out);

// Discard the packet

Packet: :free(pkt);

}

}

5	Result Visualization and Analysis

Consider NS2 as a black box with inputs as the OTcl scripts then its outputs are trace files, which can be used for network animations using NAM and/or analysis using Xgraph/GNUplot. NS2 usually generates two types of trace files: namtrace (for NAM) and trace (for Xgraph/GNUplot).

5.1	Network Animator

As mentioned in the introduction, Nam is a Tcl/TK based animation tool for viewing network simulation traces and real world packet traces. It supports topology layout, packet level animation, and various data inspection tools. Different components of NAM are shown in Figure 3 (you may enlarge it for better visibility).

The following OTcl script segment tells NS2 to generate a trace file out.nam compatible with Nam:

set nf [open out.nam v]

$ns namtrace-all $nf

After the simulation, the trace file can be animated by Nam via:

nam out.nam

# Figure 3: Network Animator (NAM) Version 1.9

During the simulation of Nam, users can watch the topology of the network and the packet movement; i.e., how each single packet (represented by a small rectangle) is moved between different nodes along links, how these packets pile up in a queue when a link is busy, and finally, if the queue is full, how the packets are dropped. The identity and status of each link can be inspected during the simulation by clicking mouse on it.

A Nam trace file is different from a non-Nam trace file in that besides of the packet trace information, it also contains topology information like nodes and links. Users can direct Nam on how to display network components.

    - Nodes Orientation

The following OTcl script segment tells Nam the layout of 4 nodes (Figure 4):

$ns duplex-link-op $n0 $n2 orient right-down

$ns duplex-link-op $nl $n2 orient right-up

$ns duplex-link-op $n2 $n3 orient right

5.1.2	Marking Flows

The following OTcl scripts tell Nam use different colors to display different data streams (Figure 4):

$udp0 set class_ 1

$udp1 set class_ 2

$ns color 1 Blue

$ns color 2 Red

    - Monitoring Queues

The following OTcl scripts is used to monitor the queue for the link from n2 to n3 (Figure 4):

$ns duplex-link-op $n2 $n3 queuePos 0.5

# Figure 4: Node Orientation, Marking Flows, and Monitoring Queues

Node n0 is transmitting blue packets. Node n1 is transmitting red packets. Both n0 and n1 are sending packets to n2 through n2. Packets are dropping in the queue for the link from node n2 to node n3.

5.2	Trace and Monitor

A non-Nam trace file traces each packet in the simulation, recording every important point in its life cycle. The following OTcl script segment tells NS2 to generate a trace file:

set ti [open out.tr v]

$ns trace-all $ti

Here is an example of a trace file:

+ 1.84375 0 2 cbr 210 --- 0 0.0 3.1 225 610

- 1.84375 0 2 cbr 210 --- 0 0.0 3.1 225 610

r 1.84471 2 1 cbr 210 --- 1 3.0 1.0 195 600

r 1.84566 2 0 ack 40 --- 2 3.2 0.1 82 602

+ 1.84566 0 2 tcp 1000 --- 2 0.1 3.2 102 611

- 1.84566 0 2 tcp 1000 --- 2 0.1 3.2 102 611

r 1.84609 0 2 cbr 210 --- 0 0.0 3.1 225 610

+ 1.84609 2 3 cbr 210 --- 0 0.0 3.1 225 610

d 1.84609 2 3 cbr 210 --- 0 0.0 3.1 225 610

- 1.8461 2 3 cbr 210 --- 0 0.0 3.1 192 511

r 1.84612 3 2 cbr 210 --- 1 3.0 1.0 196 603

+ 1.84612 2 1 cbr 210 --- 1 3.0 1.0 196 603

- 1.84612 2 1 cbr 210 --- 1 3.0 1.0 196 603

+ 1.84625 3 2 cbr 210 --- 1 3.0 1.0 199 612

Each line in the trace file starts with a event (+, -, d, r) descriptor followed by the simulation time (in seconds) of that event, from and to node, packet type and size, etc. The last field is the unique packet id. Having simulation trace file at hand, all one has to do is to transform a subset of the data of interest into a comprehensible information and analyze it. Some UNIX tools like grep and awk can do the transforming job, and some graph tools like XGraph and GNUPlot  (included in the NS2 all-in- one package) can do the analysis job.

Unlike trace, a monitor usually records the status of a network component like a single link as time elapses. The following OTcl script segment will monitor the waiting queue of a link between nodes $rl and $r2, recording the current queue length and the average queue length once for a while. After the simulation, the data recorded in trace file all.q can be analyzed by Xgraph or GNUPlot.

set redq [[$ns link $node_(r1) $node_(r2)] queue]

set tchan_ [open all.q w]

$redq trace curq_

$redq trace ave_

$redq attach $tchan_

5.3	Our Experiment with 10-Nodes Wired Network using NS2

Simulation Model Assumption,

Number of Nodes: 10

Bandwidth: 1MB

Packet Flow: n0 – n9 (red), pick rate 400KB

n1 – n6 (green), pick rate 400KB

n2 - n8 (blue), pick rate 400KB

Expectation,

Packet drop in n3 – n4

# Figure 5: NAM - Simulation of Packet Drop in a 10-node wired network

Figure 6: Xgraph - Packet Flow in a 10-node wired network

6	Ad Hoc Wireless Simulation

Our discussion so far is a simplified vision on NS2 for the ease of under- standing. Unfortunately, when we are going to simulate routing protocols for ad hoc wireless networks, we need more detailed knowledge and considerations.

6.1	Wireless Channel and MAC Layer

In a simplified vision, nodes in NS2 are connected in pairs with links. However, for wireless networks this is not true. In a wireless network, nodes first send packets to its wireless network interface (NIC), and NIC's send packets into a wireless channel shared by all wireless nodes. All wireless NIC's within the transmission range of this packet can sense the transmission, and only the specified destination will receive the packets and pass them to the upper layers in the same node.

A shared channel is more complex than a simple link. If two NIC's in the transmission range send packets simultaneously to the same channel, a collision occurs. To reduce the number of collisions, a media access control (MAC) protocol must be adopted to coordinate the activity of each nodes. Currently, most NS2 simulations for ad hoc wireless networks adopt IEEE 802.11 MAC protocol, where a node must ask for permission from all its neighbor nodes before sending a packet.

For the sake of efficiency, many researchers also modified the IEEE 802.11 protocol a bit to embed some routing specific functions. From the view of implementation, that's a lot of hacking into the NS2's source code.

6.2	Node and Routing Module

In a simple point-to-point configuration, routing layer in NS2 is a routing table implemented as a passive classifier, which decide to which link a packet is sent according to the destination address. The routing table is generated in a centralized fashion at the beginning of the simulation, or dynamically updated by an active agent on the node called routing module. For example, the distance vector (Bellman algorithm) is implemented using a routing module exchanging routing packets among neighbor nodes.

Because most ad hoc routing protocols are reactive; i.e., a route usually is not computed periodically, but rather computed on demand. However, a passive classifier cannot do this on-demand job. So implementing ad hoc routing protocols actually means a new node structure.

For example, in the widely used CMU wireless and mobility extensions to NS2, four routing protocols, including DSDV, DSR, AODV, and TORA, are implemented. However, before creating a wireless node, users must tell NS2 whether it is a wireless simulation and which routing protocol is adopted. The reason is that NS2 has different internal node structures for different routing protocols. To implement a new routing protocol also means to devise another node structure.

6.3 Mobility Simulations and Animation

Unlike static networks, ad hoc wireless network is mobile; i.e., each node changes its position once a while. The CMU extension provides new methods (saddest) for a node to change its current position. Users can either write OTcl script to tell NS2 when and where to move these nodes, or just import a scenery file generated by another program, which creates the nodes and moves them around. The move pattern of nodes during a simulation can also be recorded into a scenery file for later use.

Nam can also support the animation of wireless networks. However, it is the responsibility of each network component to record appropriate information, like the dimension of the simulation area, the position change of nodes, and the transmission range of the wireless NIC's, etc, into the Nam trace file.

6.4	Our Experiment with Wired-Wireless Network using NS2 [12]

We used a wired-wireless network environment to see how to support routing between a fixed network like the Internet and a mobile ad hoc network (MANET). As shown in Figure 7, three nodes (node 0, 1, and 2) are wired connected and formed a fixed network. Three other nodes (node 3, 4, and 5) are mobile nodes and are communication by means of wireless connection forming a MANET.

# Figure 7: Routing between fixed and mobile nodes

7	Conclusion

NS2, a discrete event driven network simulator written in C++ and OTcl, is very useful to simulate wired and wireless networks. Users are required to have a good understanding about the simulation concepts and Otcl and C++ languages to be able to use it. NS2 is open-source free software and it lacks user-friendly manuals. NS2 installation in UNIX/Linux is easy, in Windows is difficult. Once the installation is completed and users are familiar with simulation concepts, Otcl, C++, etc then NS2 is fairly easy to use.

Using NS2 for wired network simulation is less difficult. Using the existing components of Otcl is easy, but it does not do much. The methods to implement new components into NS2 using C++ are complex but powerful.

Using NS2 for ad hoc wireless network is more difficult, because that demands an extension to the network component library, more understanding to the inner structure of current components, and extension upon extensions. Once it is grasped, NS2 is very flexible, powerful, and promising tool for testing new ad hoc routing protocols.

A	Internet Resources

[1]	NS Home Page:

http://www.isi.edu/nsnam/ns/

[2]	Nam Home Page:

http://www.isi.edu/nsnam/nam/

[3]	Independent Study Report – NS2 by Fei Dai:

http://www.cse.fau.edu/~fdai/projects/is/ns2.pdf

[4]	Cygwin – Linux-like environment for Windows:

http://www.cygwin.com/

[5]	XFree86 – open-source implementation of X11:

http://www.xfree86.org/

[6]	Building NS2 on Cygwin for Microsoft Windows 9x/ME/NT/2000/XP:

http://csl.ee.iastate.edu/~cpre543/ns2/ns2_howToInstall.htm

[7]	NS2 – a Simulation Tool for Linux:

http://www.linuxjournal.com/article.php?sid=5929

[8]	Marc Greis's Tutorial (including wireless examples):

http://www.isi.edu/nsnam/ns/tutorial/index.html

[9]	NS by Example (another tutorial by Jae Chung and Mark Claypool of WPI):

http://nile.wpi.edu/NS/

[10]	OTcl Tutorial:

http://bmrc.berkeley.edu/research/cmt/cmtdoc/otcl/

[11]	CMU Wireless and Mobility Extensions to NS2:

http://www.monarch.cs.cmu.edu/cmu-ns.html/

[12]	A Study of Internet Connectivity for MANET in NS2:

http://www.telecom.lth.se/Personal/alexh/thesis.pdf